EEL 5722 - FPGA DESIGN FINAL PROJECT:

Bomberman FPGA Video Game

Contents

Introduction	3

Your Objective	4

Your VGA Module	5

Getting Started	6

Graphics Memories: Pillar Distributed Memory ROM	7

Arena & Pillars	10

Bomberman Sprites	12

Generating the Bomberman sprite Block RAM	13

Bomberma+n Module	14

Motion Timer Register	16

Sprite X,Y Coordinate Register	16

Pillar Collision Signals	17

Sprite X,Y Coordinate Registers Next State Logic	19

1. ) Animate Bomberman: Frame Timer & ROM Offset Registers	21

Instantiate Sprite ROM and Indexing	22

Outputs	23

Block Module	24

Creating the Dual Port RAM	25

Block Module IO	26

Collision Detection With Blocks	26

Block Sprite ROM & block_map RAM	27

bm_blocked	28

Bomb Module	29

IO	30

Bomb Coordinates	31

Bomb & Explosion Counters	31

2.) Bomb and Explosion FSM	32

ROM indexing and Outputs	35

Enemy Module	36

IO	36

3.) Linear Feedback Shift Register	36

4.) Enemy Finite State Machine	37

Lives and Score: game_lives Module	39

Score Display & binary2bcd Conversion	40

5.) Binary to BCD	41

Numbers ROM	42

Top Module Description	44

6. ) Bomberman Life Bar	46

7. ) Your Creative Expansion………………………………………………………………………47

Grading Rubric	47

# Introduction

This project focuses on implementing a digital system on an FPGA that plays a simple game that is similar to Bomberman.

Bomberman walks around an arena filled by an array of indestructible white pillars, and blocks made of bricks that can be cleared by laying a bomb down and exploding them. Bomberman can lay one bomb at any time, which takes a moment to detonate, and then explodes into the surrounding four tiles on screen.

The enemy in the game is a flying dragon that moves around the arena with a mind of its own. If Bomberman lays a bomb skillfully, he can hit the dragon with its explosion, thereby earning points in the game, and making the dragon move faster and more erratically.

If the dragon intersects Bomberman or Bomberman is hit by the explosion of his bomb, he will lose a life. Bomberman starts with five lives, indicated by the shade of red surrounding the arena. As Bomberman loses lives the red will become darker, until he loses all lives and it becomes black. At this point the game is over.

# Your Objective

Your goal in this project is to complete certain aspects of the game’s various modules for it to function as intended. Along the way, the functionality of the provided HDL of each core will be outlined for you to learn from and build upon. Tasks for you to complete will be coded red, with a matching number label found in the rubric at the end of this file.

# Your VGA Module

The game is displayed on a VGA monitor using the VGA module developed in the class lab assignment. Ensure that your VGA module outputs a pixel clock, as well as the (x,y) coordinate of the pixel currently being drawn on the screen, following the IO format below:

There is no frame buffer that stores all the RGB color data that is displayed on screen. Instead each element in the game (Bomberman, enemy, bomb, blocks, etc.) which has its own module receives an input of the current x/y coordinate of the pixel being drawn on screen. If the module knows that the pixel is within where its sprite should be drawn on screen it will assert an “on” signal stating that it is currently on. The top module receives this asserted signal and routes out that module’s RGB color data to the VGA driver. In this sense, the color data is dynamically drawn on screen, and the space needed to store a frame buffer is saved.

# Getting Started

The current hardware being targeted for this project is the Basys 3 FPGA board that is used in the lab.

In Vivado, create a new RTL project named bomberman_basys3, ensuring that the following parameters are selected for the Part:

Add Sources from the HDL folder in the project files folder of:

- top_module.v

- debounce_button.v

- pillar_display.v

- your VGA module

- basys3.xdc

The top_module handles IO to the FPGA peripherals, instantiates all the modules for the system and routes signals between them, passes color data from modules that generate it to the VGA driver when appropriate, and other small tasks that wrap the entire game up and make it functional.

For you, the top_module is grown as this project progresses, and you will be uncommenting module instantiations as the project proceeds. The top module as given has almost everything ready to get started with the pillar_display module.

The hierarchical view of the project so far should look as shown below:

You will see that there is one missing piece with a red question mark.

In this project we will use distributed and block memory to instantiate ROMs and a RAM.

# Graphics Memories: Pillar Distributed Memory ROM

In the graphics folder you will find images with associated coefficient files that are to be used to create prefilled memory elements.

The format of a .coe file is shown below:

with the final coefficient terminated with a semicolon.

The provided image_2_coe.py python script was used to read in each image and generate the coefficient file with 12-bit RGB color data for each pixel in the image. It is important to note that the image is read into the coefficient file in Row-major order.

In this section it will be demonstrated how to create the distributed ROM for the pillar ROM used in the pillar_display module of the following section.

From the IP Catalog, select Distributed Memory Generator.

Name the component pillar_dm.

The pillar image is 16x16, so the depth of the ROM is 256.

The width of the data is 12 bits.

Select ROM for memory type.

On the RST & Initialization tab, browse and load the pillar.coe file.

Press Ok and then Generate.

The generated interface to the memory is simple:

The address into the ROM is a, 8-bits for a depth of 256 entries, with an output spo of width 12 for the 12-bit color data stored within.

Now that the source hierarchy in Vivado is complete, take a moment to run Synthesis, Implement the design, generate the bitstream, and load it onto the FPGA.

You should see the gameplay arena centered on the monitor with an array of pillar tiles on screen.

***Note that the left most slide switch SW15 should be down to disable the reset.

# Arena & Pillars

The gameplay arena is shown below. It has dimensions of 528x432 pixels. This will be the area that Bomberman and enemies will move within.

A regular grid of stationary and indestructible pillars is formed in the arena. Each pillar has dimensions of 16x16 pixels. The arena has length and width dimensions that are evenly divisible by 16, and the display and collision detection with the grid of pillars will take advantage of this.

The module pillar_display contains combinational logic that determines where on the screen the pillars are drawn. It receives inputs for the (x, y) pixel location that is being set on screen by the VGA module. When the pixel location on screen is within where a pillar should be in the arena, the pillar_on signal is asserted, and the appropriate RGB data indexed from the pillar ROM is routed out of the module.

The VGA pixel (x, y) coordinates begin at the upper left corner of the entire screen. Since the arena is offset in the center of the entire screen with black borders, new coordinates (x_a, y_a) are generated which account for the offset and begin with (0, 0) at the top left corner of the arena. These are generated in the top module and routed in. When this offset calculation is performed, it will be described by saying that the coordinates were transformed to arena coordinates, or into the arena coordinate frame.

For a pillar to be placed in the arena in a regular grid (i.e. pillar_on output asserted):

- VGA pixel coordinates (x, y) must be in bounds of the arena.

- Both (x_a, y_a) coordinates when divided by 16 must yield an odd number (i.e. 5th bit must be 1).

This second criteria is demonstrated in the diagram below from the upper left corner of the arena where (x_a, y_a) = (0, 0).

The x_a and y_a coordinate’s lower 4 bits are used to index into the instantiated pillar ROM as shown below, where “*16” is replaced by “<< 4”.

# Bomberman Sprites

The Bombermans sprites used to animate the main character on screen are stored in block RAM on the FPGA to form a ROM. The coefficients are derived from a 16x250 image, where each of the 10 sprite frames is individually 16x25 pixels. A python script reads in this image and flattens the image coefficients to output a .coe file where the coefficients are in the row-major order format ({row_0}, {row_1}, … , {row_299}) where each row contains the 16 pixel values across the row from left to right.

Indexing into the ROM to a specific sprite frame is done by using an offset multiple of 25 to get down to the start of the frame.  Where the Bomberman sprite is on screen and current pixel coordinates of the VGA pixel being drawn on screen are related to fully index into the ROM:

.addra( (x – x_b) + (y – y_b + offset) << 4)

= .addra( (x – x_b) + {(y – y_b + offset), 4’d0})

Where (x, y) are VGA pixel coordinates being drawn on the screen, (x_b, y_b) are Bomberman’s sprite coordinates denoting the upper left of the sprite (to be detailed later), and offset is the value that determines where to start indexing into the ROM or what sprite to draw on screen.

# Generating the Bomberman sprite Block RAM

In the IP catalog open the Block Memory Generator

Under the basic tab yype in bm_sprite_br for the component name.

Select Single Port ROM for Memory Type, with native interface.

Under the Port A Options tab, use a Width of 12, and Depth of 16x250 = 4000.

Fron the Other Options tab, check “Load Init File” and browse for the bomberman_sprites_10.coe file. Check to fill remaining memory locations with 0.

Press OK and generate the ROM.

The interface has a clock input clka which should be connected to the master clock. The address input addra addresses into the 4000 entry ROM, and the output douta outputs the 12-bit color data.

Add the source bomberman_module.v to the project.

In the top_module uncomment the instantiation of the bomberman_module.

# Bomberman Module

The Bomberman module takes care of several tasks:

- Display Bomberman on screen based on location coordinates of the sprite.

- Update Bomberman’s location in response to user input and collision detection.

- Animate Bomberman while he walks.

- Index into Bomberman sprite ROM and output RGB color data appropriate for the current x/y VGA pixel location.

- Output to other modules Bomberman’s location to be used for collision detection.

This is the largest and most complex module in the whole game. It will be your job to implement the timing and indexing into the sprite ROM to properly animate the character on screen.

IO

Inputs L, R, U, D come from the debounced user input of pushbuttons and tell this module which direction to move Bomberman on screen.

Input cd comes from the top_module where the current direction is registered from the inputs L, R, U, D.

Input bm_blocked is a signal from the block_module that is asserted when Bomberman would collide with a block if he moved from his current location in his current direction and is used for collision detection. The block_module and bm_blocked will be discussed in detail later.

The input gameover is a signal asserted by the game_lives module when the game is over and is used to shut off Bomberman’s motion in this module.

Output bomberman_on is used in the top_level module to coordinate when the VGA pixel drawing on screen is located within Bomberman’s sprite on screen, and therefore should display the Bomberman module’s RGB output signal, rgb_out.

The output bm_hb_on is a signal that is asserted when the VGA pixel on screen is within Bomberman’s hitbox that is used for collision detection. This signal is used in the game_lives module, as discussed later.

The output x_b, y_b is bomberman’s current position on screen.

## Motion Timer Register

To move the sprite of Bomberman across the screen in response to user input, its location coordinate registers (x_b, y_b) are incremented or decremented. For instance, if the user input R is asserted, Bomberman’s sprite location should move right, with x_b incrementing. For this motion to be on human appreciable time scales, a timer is used to count for a specific period before incrementing the sprite location by one step.

When a user direction input is asserted, the motion timer register counts from 0 to TIMER_MAX, upon which a “tick” signal is asserted and the register resets to 0. When no user direction input is asserted the timer resets to 0.

The motion_timer_tick signal is then used to update the sprite location at a normal rate on screen. For instance, if the FPGA runs at 100 MHz, and TIMER_MAX = 1,200,000, then the period of the counter is 0.012 seconds, which means in one second, the sprite would move 83 pixels. This is arbitrary and can be adjusted until Bomberman moves at a speed that is appropriate for gameplay.

## Sprite X,Y Coordinate Register

Two registers are inferred to hold the x and y coordinates of the Bomberman Sprite on the screen. The coordinate location is defined as the upper-left corner of the sprite.

## Pillar Collision Signals

Bomberman’s sprite location should move when the user input signals a direction to move, the motion timer register has ticked, and the sprite can move in that direction. The sprite cannot move if Bomberman would run into a pillar, Bomberman would leave the arena, or Bomberman would run into a block.

When developing a model for constraining the sprite motion across the arena to stay between pillars, a 16x16 hitbox is placed at the bottom of the sprite and is constrained to not intersect with where a pillar is drawn on screen, known as the x and y pillar space.

There are four signals, p_c_up, p_c_down, p_c_left, and p_c_right, which are asserted when a pillar collision would happen if the sprite moved one pixel in the corresponding direction. These asserted signals tell the sprite x_b_reg/y_b_reg coordinate registers to not increment or decrement the location if it would mean colliding with a pillar.

For instance, suppose the Bomberman sprite is facing down, and the hit box is right against a pillar below:

In this state, p_c_down would be asserted, and the sprite’s y location would not be allowed to increment and move down a pixel.

The x coordinates of the left/right edge of the box, and y coordinate below the bottom edge of the box can be tested for being within the space of a pillar (5th bit == 1, as explained in Arena & Pillars section).

If either left or right edge coordinates are within the pillars x space, and the y coordinate below the hitbox bottom are within the pillars y space, then the p_c_down signal is asserted.

The state shown below on the left would assert p_c_down signal, as the left edge x coordinate is within the pillar x space, and the y coordinate below the bottom edge is in the pillar y space.

The state shown below on the right would not assert p_c_down. While the y coordinate below the bottom edge of the hit box is in the pillar y space, neither the left or right edge of the hit box are within the pillars x space.

The p_c_down wire is assigned to the conditional statement:

Where the signals being compared in this conditional statement are defined as:

Where:

The y_b_bottom signal is the current sprite y coordinate, - 32 to translate into arena coordinates (see section Arena & Pillars), + 25 + 1 to bring the value down to the y coordinate below Bomberman’s hitbox.

The x_b_hit_l and x_b_hit_r are the x coordinates of the left and right sides of the hit box derived from the Bomberman sprite x coordinates, transformed into arena coordinates.

The pillar collision signals for up, left, and right are derived in a similar manner and are listed in the bomberman_module source file in the Pillar Collision Signals section.

The x_b_hit and y_b_hit signals are further constrained as shown below, and are used in the final collision detection to avoid the corner case where Bomberman walks into block when going around a pillar.

## Sprite X,Y Coordinate Registers Next State Logic

The pillar collision signals work to constrain the Bomberman sprite 16x16 hitbox to remain within the 16x16 gaps between pillars. Without this constraint it would be difficult for a user to accurately place the 16x16 hitbox between the 16x16 gaps to change from moving either up/down to left/right or vice versa.

For example, assume the Bomberman sprite is moving as shown in the image below, down a column. For the Bomberman sprite to move either left or right between the pillars, its hitbox must be aligned as shown. If it is not aligned with the left/right empty rows, a pillar collision signal will be asserted, and the Bomberman Sprite will not be allowed to move left/right. This makes sense, as the sprite hitbox is constrained to never collide with a pillar.

To make moving around pillars in the arena easy, Bomberman will slide adjacent to the pillar currently being run into to the nearest gap. In the instance shown below, when Bomberman walks down against the pillar, he will slide right until his hitbox clears the gap and can move down the column.

In short, this will allow Bomberman to automatically move around pillars when walking into them.

Considering just the next state logic that would allow y_b for Bomberman moving down:

The first line checks if the game is not over, if Bomberman is not blocked by a block (as explained later), and the motion tick has been asserted. The second line allows for Bomberman to normally move down an empty column. If the current input is down (CD_D), and there is no expected pillar collision, and the current coordinate isn’t at the bottom boundary of the arena, and the motion timer register ticks for the sprite update time delay, then increment the register by one and move Bomberman down.

The third line of logic allows for Bomberman to move down and around the bottom of a pillar if moving right into the bottom of a pillar: if the current input direction is right, and there is an expected pillar collision if moving right, and the top of the hit box is touching the pillar, and a motion timer tick, then move down, which will help Bomberman go down and around the pillar and move right.

The fourth line of logic allows for the same situation of going down and around the bottom of a pillar if moving left into it.

The next state logic for x_b and y_b to be incremented or decremented follows the same format:

## 1. ) Animate Bomberman: Frame Timer & ROM Offset Registers

When Bomberman is not walking, the frame that is displayed on screen should be of him standing in the last direction input by the user, held in the current direction register input from the top module. This would be frames shown below, labelled U1, R1, D1, and frame R1 mirrored horizontally for left.

When any user direction input is asserted, a timer register called frame_timer_reg should count up, and cycle through a timing period of T = 50000000 cycles, with 4 sub periods of T/4. During the four sub periods, the frames drawn on screen should cycle between 1, 2, 1, 3, for the specified direction. This animates Bomberman’s movement on screen with a walking animation.

✎ Design the frame_timer_reg register and frame_timer_next next-state logic to satisfy the above requirements.

A rom offset register called rom_offset_reg should hold predefined offset values that are multiples of 25 up to 225, which are used to index into the ROM to the start of a particular frame to be displayed.

The next state logic for the rom_offset_reg should use the current direction register and frame timer register to determine which offset to use to draw the correct frame on screen.

✎ Design the rom_offset_reg  register and rom_offset_next next-state logic to satisfy the above requirements.

With the above requirements undefined in the source, the frame_time_reg and rom_offset_reg are currently seen by synthesis as being unused, and are therefore automatically tied to 0, which causes the first frame in the ROM to be constantly displayed.

## Instantiate Sprite ROM and Indexing

The block RAM module for the Bomberman sprite, bm_sprite_br, is instantiated, with clk input routed to clka, indexing address br_addr routed to addra, and color output rgb_out taken from douta.

The indexing into the rom uses VGA x/y pixel coordinates, Bomberman coordinates x_b_reg / y_b_reg and the rom offset reg defined above. When the current direction is left, the indexing done in the x dimension is mirrored.

## Outputs

The module outputs are assigned at the end.

Bomberman location register is outputted to be used for collision detection in the block_module.

The signal bomberman_on is asserted when the current VGA pixel location x/y is located within the sprite drawn on screen and is used in the top module to route out the RGB color info to the VGA module when it is to be displayed on screen.

The signal bm_hb_on is asserted when the current VGA pixel location is located within the hitbox location on screen and is used in the game_lives module for hit detection with the enemy and explosion, as explained later.

With the Bomberman module up and working, Bomberman is able to easily walk around the arena. Let’s now explore how to place blocks on the screen that Bomberman will eventually contend with using bombs.

# Block Module

In the game arena there are blocks that are placed in empty 16x16 spots which block Bomberman and can be destroyed by a bomb.

Since each block is 16x16, the arena can be divided evenly into an array of 33 columns and 27 rows. This will be referred to as the arena block map coordinates, or ABM coordinates. To transform x/y coordinates on the monitor to ABM coordinates, subtract the offset of the top left location of the arena to make the top left pixel of the arena 0. Next divide by 16, or use all bits above the lower 4, and this will give a number that fits within the ABM coordinate frame.

A dual port RAM is used to store 32x26 1-bit values that determine if a block is located at each spot in the arena. By converting the VGA x/y coordinates to ABM coordinates, the read index into the ROM is formed as:

where

Add the source file block_module.v to the project and uncomment its instantiation in the top module.

## Creating the Dual Port RAM

The dual port RAM used to store ABM locations is prefilled by the block_map.coe file. The RAM contains 1-bit values for each ABM location, with 1 being a bomb, and 0 being empty. This coefficient file can be changed to create a new arrangement of blocks in the arena. It can be generated using the block_map_2_coe Python code which works on the block_map.png image, where red pixels denote blocks on the map.

block_map.png

To generate the dual port RAM, open the Distributed Memory Generator. Name the component block_map. Use a data width of 1, and depth of 896. Include the coefficients file and generate.

Use the directions of creating a distributed memory ROM to create the ROM of the block sprite using the component name block_dm from the block.coe file.

## Block Module IO

Inputs x/y are VGA pixel coordinates being drawn on screen.

Inputs x_a/y_a are VGA pixel coordinates in arena coordinate reference frame.

Input cd is registered current direction of Bomberman as given by control input.

Inputs x_b/y_b are coordinates of Bomberman on the screen, used for collision detection with blocks.

Input waddr is the write address to the block_map RAM, used by the bomb_module’s explosion tiles to clear blocks in RAM.

Input we is the write enable signal to the dual port RAM block_map.

Output rgb_out is the RGB color data for blocks to be drawn on screen when block_on is asserted.

Output signal bm_blocked is asserted when it is calculated using x_b/y_b and cd that a collision with Bomberman and a block is to occur.

## Collision Detection With Blocks

To determine if Bomberman would collide with a block, moving in his current direction, signals are defined that represent the pixel location of the different edges of Bomberman’s hitbox moving in the different possible directions. These are formed below:

An important fact about the dual port RAM used to represent where blocks are on screen is that there is only one read and one write port. Reading for the purpose of displaying blocks on screen must be separate from reading for the purpose of collision detection. For this reason, the read address into the block RAM, raddr, is specified depending on if the display_on signal is asserted, meaning pixels are being drawn on screen, or deasserted, meaning the VGA module is not drawing on screen.

When the display_on signal is asserted and the VGA x/y pixel coordinates are within the arena bounds, the read address is formed by using the x/y coordinates converted into ABM coordinates to index to the correct block in the RAM. When the display_on signal is deasserted, the read address is formed depending on the current direction, using the signals defined above, converted to ABM coordinates.

## Block Sprite ROM & block_map RAM

The block sprite distributed memory block_dm is instantiated and indexed using the VGA pixel x_a/y_a arena coordinates lower 4 bits, as they repeat every 16 pixels for each 16x16 block.

The block_map dual port RAM is instantiated, with data_in tied to 0 in order to clear blocks when the waddr write address is set and the we write enable is asserted by the bomb_module’s explosion tiles, as explained later.

## bm_blocked

The bm_blocked output is asserted when it is calculated using x_b/y_b and cd that a collision with Bomberman and a block is to occur. The raddr into the block_map RAM is set using the cd and signals formed above such that the contents of the RAM is tested to be 1, indicated a block is in Bomberman’s path. The blocked_reg registers this blocked status and is asserted when the display is off and the data from the RAM is 1.

The block_on output is asserted when the display is on, and the data_out from the RAM indicates that the VGA x/y pixel coordinates are within a block on screen.

With the block module up and working, Bomberman is stuck in the upper left-hand corner of the screen. Time to explore how he will blast through.

# Bomb Module

Add to the project the source file bomb_module.v and uncomment its instantiation in the top module. Generate the distributed memory ROM for the bomb.png image, naming the component bomb_dm. Generate the block memory ROM for the explosions.png image, naming the component explosions_br.

The purpose of the bomb module is to drop a bomb to the ABM tile nearest to the center of Bomberman’s hitbox. This bomb remains on screen for a short period, and then explodes. The explosion encompasses the tile of the bomb, as well as the tiles to the left, right, top, and bottom. The explosion does not affect pillars or the walls and can destroy blocks. A carefully timed explosion will hit the enemy but can also hit Bomberman.

## IO

Input includes VGA pixel coordinates in arena coordinate frame x_a/y_a, Bomberman’s current direction cd, Bomberman’s location x_b/y_b , bomb input button signal A, and the gameover signal used to stop bombs from being placed when the game is over.

Outputs include RGB color data and on signals for the bomb and explosion tiles on screen, the write address and write enable to the block_map RAM in the block_module, and finally post_exp_active which is asserted when the explosion is active on screen and is used in the enemy_module, as discussed later.

## Bomb Coordinates

The bomb coordinates x_bomb_a/y_bomb_a are derived from Bomberman’s coordinates, and roughly translates to the center of the hitbox.

In the image below, Bomberman’s coordinates are defined by the upper left pixel of the frame (red), and the bomb coordinates are defined by the pixel marked with an x.

## Bomb & Explosion Counters

The bomb_counter_reg counts for a time period until it reaches the max value BOMB_COUNTER_MAX, upon which it will reset to 0. The counter is initiated by the bomb_active_reg signal which is part of the FSM described below and is asserted when the bomb is to be active on screen, during a state of the FSM.

The explosion counter works the same way, counting to a max value, and being initiated by a registered signal which is asserted when the explosion is to be active on screen, in a state of the FSM.

## 2.) Bomb and Explosion FSM

The state machine for the bomb and explosion sequence is shown below as an ASMD chart, where squares represent states, diamonds represent a condition to be tested, and an oval represents data that is changed in response to a condition.

✎ Implement the state machine described below. Use the registers and state values defined in the bomb_module.

Before the user presses the bomb button A, there is no bomb or explosion on screen, and the FSM state is no_bomb. If the user presses A and the game is not over, assert the bomb_active_reg which allows the bomb counter to run, set the bomb coordinates register to the current location of the middle of Bomberman’s hitbox, x_bomb_a / y_bomb_a, and go to the bomb state.

In the bomb state, wait for the bomb_counter_reg to reach the max value (keeping the bomb on screen for a period), then deassert the  bomb_active_reg, assert the exp_active_reg which allows the explosion counter to run, assert the block_we_reg to allow writing into the block_map RAM for explosions to clear blocks, and go to the exp_1 state.

In the exp_1 state, the ABM coordinate to the left of the bomb is routed to the write address of the block_map RAM so it will set that location to zero, clearing a block if it were there. This action only happens if the x coordinate of the bomb is not zero, thus making sure not to index into the RAM wrongly where there is a wall. The state machine then goes to the exp_2 state.

The exp_2, exp_3, exp_4 states are similar in that they set the write address to the block_map RAM for the ABM coordinates to the right, top, and bottom of the bomb, clearing those coordinates in the RAM as well. Each state goes directly to the next, until exp_4 goes to the post_exp state.

In the post_exp state, the post_exp_active output is asserted, which is used in the enemy_module, as discussed later. The state then allows the explosion counter to count up the max value (keeping the explosion on screen for a period), then set the exp_active_reg  and block_we_reg to 0. The FSM then goes back to the no_bomb state.

One aspect that is left out of this diagram that is in the final code is to test for the edges of walls when indexing into waddr, i.e. if using bomb_x_reg-1, make sure bomb_x_reg != 0.

## ROM indexing and Outputs

The output bomb_on is asserted when the VGA pixel in ABM coordinates (x_a[9:4], y_a[9:4]) are equal to the bomb ABM coordinates, and the bomb_active_reg is asserted, meaning the bomb is active on screen. This signal is used in the top_module to route RGB output from the bomb_module to the VGA driver when the current VGA pixel is within where a bomb should be on screen.

The assignment for exp_on is similar but is asserted for when the VGA pixel is in the center of the explosion, as well as 4 surrounding tiles.

The ROMs are instantiated and indexed into, and the block_w_addr and block_we are assigned to the registers controlled in the FSM data path.

# Enemy Module

The enemy module creates a single enemy dragon on screen that moves around the arena. The dragon will randomly change its direction and can fly over blocks contained within the arena. When the enemy’s sprite overlaps an explosion tile on screen, it will be hit. When hit it will stop, look distressed, and then speed up.

This module follows a form that is nearly identical to the bomberman_module for sprite ROM indexing.

Add to the project the source file enemy_module.v and uncomment its instantiation in the top module. Generate the block memory ROM for the enemy_sprites.png image, naming the component enemy_sprite_br.

## IO

The module has inputs for VGA pixel coordinates, Bomberman’s coordinates, and bomb ABM coordinates. Inputs also include exp_on which is asserted when the VGA pixel is within an explosion tile on screen and is used to determine if an explosion is on at the same time as the enemy, denoting a collision or enemy hit. The post_exp_active signal controlled from the bomb_module FSM is used to hold the enemy still when it is hit by an explosion for the entire duration of the explosion. The output enemy_hit is asserted when the enemy is hit by an explosion and is used in the game_lives module to increment a score counter.

## 3.) Linear Feedback Shift Register

The enemy will need a pseudorandom register to achieve a random walk across the arena by taking random direction changes with a random probability.

✎ Design a Maximal Length 16-bit Linear Feedback Register module with the following specifications:

- clk / rst inputs

- write enable input

- 16-bit write data input that sets the shift register to a new starting value.

- 16-bit output

- Maximal length output sequence of 216-1 unique values.

Use the following module format:

## 4.) Enemy Finite State Machine

✎ Implement the state machine described below. Use the registers and state values defined in the enemy_module.

The enemy begins centered in an ABM tile with a current direction stored in e_cd_reg. The enemy moves 16 pixels in the current direction, as tracked by move_cnt_reg. The enemy is then at a new ABM tile. With a 1/8 probability, determined by the 3 bits in random[4:2] being 000, the e_cd_reg is set equal to the lower 2 bits random[1:0] and the enemy gets a new direction. It is then determined if this new direction is legal, meaning it will not allow the enemy to pass through the arena walls, or pass through a pillar. If it is illegal, a new random value is chosen until it is legal.

The ASMD for the FSM for the enemy module is shown below:

When the signal enemy_on (asserted when VGA pixel is within enemy sprite on screen) and the input signal exp_on are simultaneously asserted, it means that the sprites of the enemy and explosion overlap on screen and the enemy has been hit. The FSM state will go to the exp_enemy state where it will remain until the post_exp_active signal from the bomb_module is deasserted, meaning the explosion on screen is finished. During the time where the enemy remains hit, the final sprite of the enemy is shown, and the enemy does not move.

However, every time an enemy is hit, it should start moving faster and there should be higher probability of more random movement.

# Lives and Score: game_lives Module

Add to the project the source file game_lives.v and uncomment its instantiation in the top module.

The game_lives module maintains a “lives” counter, beginning at 5, and decrements each time the enemy collides with Bomberman’s hitbox on screen, or when Bomberman collides with an explosion tile on screen. The module outputs a background RGB value which is correlated to the lives counter, going from red to black, which gives an indication of lives left. The output gameover is asserted when the lives counter reaches zero and is used in the bomberman_module to stop Bomberman from moving and dropping bombs when the game is over.

An invisibility counter begins counting when Bomberman is initially hit by an enemy or explosion and is used to prevent successive damage from being taken for a short period.

The next-state logic for the invisibility counter resets it to 0 if it reaches the max, increments the counter if the value is above 0 and below the max, and begins counting by setting the value to 1 when the explosion or enemy overlaps Bomberman’s hit box.

# Score Display & binary2bcd Conversion

Add to the project the source file score_display.v and uncomment its instantiation in the top module. Add to the project the ROM template source file numbers_rom.v.

The score_display module keeps track of a score counter, incremented by 10 each time the enemy is hit by an explosion, counting from 0 to 9999. The score is displayed on screen using 4 digits. The score, stored as a binary value in score_reg, is converted by the binary2bcd module to four binary-coded decimal values that are displayed on screen.

The enemy_hit input is asserted by the enemy_module when the enemy is hit by an explosion and its FSM is within the post_exp state. A positive edge detection circuit is designed, and the positive edge of enemy_hit is used to increment score_reg by 10, if it remains below 9999.

## 5.) Binary to BCD

✎ Implement a module that converts a 14-bit binary number to four binary coded decimal numbers that represent the number.

The binary2bcd module converts a binary value of score_reg (0 to 9999) into a binary coded decimal format where four binary values represent the decimal numbers to be displayed on screen. For instance the binary number 100111000011112 (999910) would be represented as for four binary 9 values: 100110  100110 100110 100110.

While a fully combinational implementation would suffice, your implementation should be algorithmic and use a state machine, should take in a 14-bit binary number, and output four BCD values that represent it. A testbench along with correct waveforms and test values for inputs within the range of 0 to 9999, including the bounding values 0 and 9999, is required.

The module should follow the following IO format:

The module takes in a 14-bit value in, and a start signal to initialize the conversion. Since the conversion is used on numbers displayed on screen and occurs quickly when a score is changing, intermediate output values that are not valid are not an issue, so an output signal indicating the conversion has finished is not necessary. Both count and state are left unconnected in the instantiation of the module but are used in the testbench to display progress during the algorithmic conversion for testing purposes.

The binary2bcd module is instantiated in the score_display module.

## Numbers ROM

The numbers ROM is a ROM holding 1-bit values, where values coded 1 represent the font. The numbers_rom is a ROM template that uses block RAM, and follows the format detailed in the Xilinx XST guide to inferring memories. The ROM is its own module and is instantiated the same way.

The row and col registers are used to index into the ROM by specifying the row and column of the pixel in the image, with the origin being in the upper left of the image.

The score is displayed roughly in the top center of the screen. The constants NUM_TOP and NUM_BOTTOM store the top and bottom y coordinates of the score on screen. The constants A – E represent the five boundaries of the four BCD digits on screen and are used to define the indexing into numbers_rom depending on where the VGA pixel x/y is and the current BCD value.

An always statement is used to set the score_on, row, and col register values depending on what the current BCD registers are, and if the VGA pixel location is within the BCD value on screen. If y is within the score location on screen, and x is within A and B, i.e. is within bcd3 of the score, the col register is formed as x-A and the row register is formed as (y – 16)*bcd3*16, where bcd3 is a value from 0 to 9 and when multiplied by 16 will index to the beginning row of the value in the numbers_rom. If the color_data output of the ROM is 1, then the score_on signal is asserted, letting the top_module know to draw pixel data on screen for the font of a number.

# Top Module Description

The top module handles IO to the FPGA peripherals, instantiates all the modules for the system and routes signals between them, passes color_data from modules that generate it to the VGA driver when appropriate, and other small tasks that wrap the entire game up and make it functional.

IO are shown below with comments to indicate purpose:

The asynchronous reset button is synchronized to avoid metastability issues.

Inputs from 5 tactile pushbuttons are debounced using the provided debounce_button module to prevent glitches during gameplay that allow input to break rules of the game.

Up next are constants and various wires and regs used between module instantiations and in the rest of top.

The x_a/y_a VGA pixel coordinates in arena coordinate frame are generated and passed to modules that utilize them.

The current direction is a sampling of the directional input buttons and keeps a registered state of the direction of Bomberman.

The wall_on signal is asserted when the VGA pixel coordinates are outside of the arena.

Next up are module instantiations.

The rgb_reg contains the RGB data to route to the RGB DAC. The next-state logic uses the pixel_tick pixel clock signal from the VGA module to determine when to update the RGB output data for a new pixel location. A priority routing network uses a module’s asserted “on” signal to route out its RGB data. The priority is given to RGB data to display RGB data in the correct way, i.e. Bomberman walking over a bomb, rather than under it. RGB data for Bomberman, enemy, bomb, and exp are tested to not be the arbitrary background color for the respective image before routing the RGB data out. If the score_on signal is asserted, meaning the font of the score is to be displayed, the RGB output is set to all on to display white numbers. If no “on” signals or wall_on are asserted, then the background green color is displayed. RGB data in rgb_reg is only output to the RGB DAC output rgb when the display_on signal is asserted.

## 6. )Bomberman Life Bar

The game_lives module already takes care of calculating the lives of bomberman and displays a suitable background color. However, the experiment requires you to implement a 20 pixel * 8 pixels (width and height respectively) red colored life bar for the bomberman. This bar should be displayed in the top right corner. Since the bomberman has 5 lives, a portion of the red bar should be filled with black color each time the bomberman receives a hit. Once all the 5 lives are lost, the entire life bar turns black. No coe file or pre-defined logic is provided for the red bar in the source code. You should write your own custom logic.

# 7. ) Your Creative Expansion

Now that you have completed the basic form of the game, it is time for you to choose your expansion of the game from the options below:

- Modify the game for a two-player mode. Two Bomberman characters are playable on screen and are identifiable by their color by changing the white color of the original for the second player. You could have the game be a match of who can get the highest score in a certain time period, or a battle royale between the two players.

- Modify the game to have have a pause functionality. One additional input can be added to pause the game. Pressing the pause button should freeze/unfreeze the movement of both the bomberman and the enemy.

- Modify the game to have a soundtrack and sound effects. Use the audio output from the FPGA development board. Add a title screen to the game with title screen music. Add music to be played during game play. Add sound effects for a bomb being placed, bomb exploding, Bomberman being hit, and an enemy being hit.

# Grading Rubric

- Animate Bomberman                                    20%

- Bomb and Explosion FSM                              25%

- Linear Feedback Shift Register                       5%

- Enemy Finite State Machine                          25%

- Binary to BCD                                                10%

- Animate Life Bar					     15%

- Your Creative Expansion                               20%

Total                                                                     100% + 20%